Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#2)

Combinational Circuits Using Verilog

Student Name: HAIDER IRFAN Roll No: 24I-6505 Section: A

Date performed: August 25, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task, I made a half adder and used two of those modules to make a full adder, which is a bottom-up approach. The second half adder’s inputs were an external cin and the sum of the first half adder. The outputs gave a final sum and a carry out. These were simulated using a testbench and also designed on Logisim.

Code:

GATE LEVEL MODELLING: module ha1(sum,carry,a,b); input a,b; output sum,carry; xor(sum,a,b); and(carry,a,b); endmodule module ha2(sum,carry,a,b); input a,b; output sum,carry; xor(sum,a,b); and(carry,a,b); endmodule module fa(SUM,cout,a,b,cin); input a,b,cin; output SUM,cout; ha1 h1(sum1,c1,a,b); ha2 h2(SUM,c2,sum1,cin); or(cout,c1,c2); endmodule module testbench(); // dont pass any parameter in test bench reg a; reg b; reg cin; wire SUM; wire cout; fa uut(SUM,cout,a,b,cin); initial begin a = 0; b = 0; cin = 0; #50 a = 0; b = 0; cin = 1; #50 a = 0; b = 1; cin = 0; #50 a = 0; b = 1; cin = 1; #50 a = 1; b = 0; cin = 0; #50 a = 1; b = 0; cin = 1; #50 a = 1; b = 1; cin = 0; #50 a = 1; b = 1; cin = 1; end endmodule DATAFLOW LEVEL MODELLING: module ha1(sum,carry,a,b); input a,b; output sum,carry; assign sum = a ^ b; assign carry = a & b; endmodule module ha2(sum,carry,a,b); input a,b; output sum,carry; assign sum = a ^ b; assign carry = a & b; endmodule module fa(SUM,cout,a,b,cin); input a,b,cin; output SUM,cout; ha1 h1(sum1,c1,a,b); ha2 h2(SUM,c2,sum1,cin); assign cout = c1 + c2; endmodule module testbench(); // dont pass any parameter in test bench reg a; reg b; reg cin; wire SUM; wire cout; fa uut(SUM,cout,a,b,cin); initial begin a = 0; b = 0; cin = 0; #50 a = 0; b = 0; cin = 1; #50 a = 0; b = 1; cin = 0; #50 a = 0; b = 1; cin = 1; #50 a = 1; b = 0; cin = 0; #50 a = 1; b = 0; cin = 1; #50 a = 1; b = 1; cin = 0; #50 a = 1; b = 1; cin = 1; end endmodule

Logisim Circuit:

Output:

Task 2:

Description:

In this task, I had to make a 3x8 decoder for which I made a truth table first and found out the minterm combination for each output. I then replicated the combinations in Verilog with both gate level and dataflow level modelling. Similarly, the circuit was constructed in Logisim as well.

Code:

GATE LEVEL MODELLING: module decoder(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c); input a,b,c; output y0,y1,y2,y3,y4,y5,y6,y7; wire nota; wire notb; wire notc; not(nota,a); not(notb,b); not(notc,c); and(y0,nota,notb,notc); and(y1,nota,notb,c); and(y2,nota,b,notc); and(y3,nota,b,c); and(y4,a,notb,notc); and(y5,a,notb,c); and(y6,a,b,notc); and(y7,a,b,c); endmodule module testbench(); // dont pass any parameter in test bench reg a; reg b; reg c; wire y0; wire y1; wire y2; wire y3; wire y4; wire y5; wire y6; wire y7; decoder uut(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c); initial begin a = 0; b = 0; c = 0; #50 a = 0; b = 0; c = 1; #50 a = 0; b = 1; c = 0; #50 a = 0; b = 1; c = 1; #50 a = 1; b = 0; c = 0; #50 a = 1; b = 0; c = 1; #50 a = 1; b = 1; c = 0; #50 a = 1; b = 1; c = 1; end endmodule DATAFLOW LEVEL MODELLING: module decoder(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c); input a,b,c; output y0,y1,y2,y3,y4,y5,y6,y7; wire nota; wire notb; wire notc; assign nota = ~a; assign notb = ~b; assign notc = ~c; assign y0 = nota & notb & notc; assign y1 = nota & notb & c; assign y2 = nota & b & notc; assign y3 = nota & b & c; assign y4 = a & notb & notc; assign y5 = a & notb & c; assign y6 = a & b & notc; assign y7 = a & b & c; endmodule module testbench(); // dont pass any parameter in test bench reg a; reg b; reg c; wire y0; wire y1; wire y2; wire y3; wire y4; wire y5; wire y6; wire y7; decoder uut(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c); initial begin a = 0; b = 0; c = 0; #50 a = 0; b = 0; c = 1; #50 a = 0; b = 1; c = 0; #50 a = 0; b = 1; c = 1; #50 a = 1; b = 0; c = 0; #50 a = 1; b = 0; c = 1; #50 a = 1; b = 1; c = 0; #50 a = 1; b = 1; c = 1; end endmodule

Logisim Circuit:

Output:

Task 3:

Description:

In this task, I had to make a 8x1 multiplexer which used three select lines to choose one out of 8 possible outputs. The combination of the select lines selected a different output each time. This was represent in Logisim and also simulated in Modelsim. The inputs were given supposed values to check whether the multiplexer worked properly.

Code:

GATE LEVEL MODELLING: module multiplexer(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c,z); input a,b,c,y0,y1,y2,y3,y4,y5,y6,y7; output z; wire nota; wire notb; wire notc; not(nota,a); not(notb,b); not(notc,c); wire w0,w1,w2,w3,w4,w5,w6,w7; and(w0,y0,nota,notb,notc); and(w1,y1,nota,notb,c); and(w2,y2,nota,b,notc); and(w3,y3,nota,b,c); and(w4,y4,a,notb,notc); and(w5,y5,a,notb,c); and(w6,y6,a,b,notc); and(w7,y7,a,b,c); or(z,w0,w1,w2,w3,w4,w5,w6,w7); endmodule module testbench(); // dont pass any parameter in test bench reg a; // select lines reg b; reg c; reg y0; reg y1; reg y2; reg y3; reg y4; reg y5; reg y6; reg y7; wire z; multiplexer uut(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c,z); initial begin y0 = 0; y1 = 0; y2 = 1; y3 = 1; y4 = 0; y5 = 0; y6 = 1; y7 = 0; a = 0; b = 0; c = 0; #50 a = 0; b = 0; c = 1; #50 a = 0; b = 1; c = 0; #50 a = 0; b = 1; c = 1; #50 a = 1; b = 0; c = 0; #50 a = 1; b = 0; c = 1; #50 a = 1; b = 1; c = 0; #50 a = 1; b = 1; c = 1; end endmodule DATAFLOW LEVEL MODELLING: module multiplexer(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c,z); input a,b,c,y0,y1,y2,y3,y4,y5,y6,y7; output z; wire nota; wire notb; wire notc; assign nota = ~a; assign notb = ~b; assign notc = ~c; wire w0,w1,w2,w3,w4,w5,w6,w7; assign w0 = y0 & nota & notb & notc; assign w1 = y1 & nota & notb & c; assign w2 = y2 & nota & b & notc; assign w3 = y3 & nota & b & c; assign w4 = y4 & a & notb & notc; assign w5 = y5 & a & notb & c; assign w6 = y6 & a & b & notc; assign w7 = y7 & a & b & c; assign z = w0 + w1 + w2 + w3 + w4 + w5 + w5 + w6 + w7; endmodule module testbench(); // dont pass any parameter in test bench reg a; // select lines reg b; reg c; reg y0; reg y1; reg y2; reg y3; reg y4; reg y5; reg y6; reg y7; wire z; multiplexer uut(y0,y1,y2,y3,y4,y5,y6,y7,a,b,c,z); initial begin y0 = 0; y1 = 0; y2 = 1; y3 = 1; y4 = 0; y5 = 0; y6 = 1; y7 = 0; a = 0; b = 0; c = 0; #50 a = 0; b = 0; c = 1; #50 a = 0; b = 1; c = 0; #50 a = 0; b = 1; c = 1; #50 a = 1; b = 0; c = 0; #50 a = 1; b = 0; c = 1; #50 a = 1; b = 1; c = 0; #50 a = 1; b = 1; c = 1; end endmodule

Logisim Circuit:

Output: