Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#3)

Behavioral-Level Verilog

Student Name: HAIDER IRFAN Roll No: 24I-6505 Section: A

Date performed: September 1, 2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task, I made a 4-bit binary adder-subtractor. The inputs numbers and output were 4-bit binary numbers, while the control input decided whether the numbers were added or subtracted, with 1 representing subtraction and the 0 representing addition.

Code:

module addsub(a,b,control,c); input [3:0] a,b; input control; output reg [3:0] c; always @ * begin if (control == 0) c = a + b; else if (control == 1) c = a - b; end endmodule module testbench(); reg [3:0] a,b; reg control; wire [3:0] c; addsub uut(a,b,control,c); initial begin a = 4'b0000; b = 4'b0101; control = 0; #50; a = 4'b0110; b = 4'b0011; control = 1; #50; end endmodule

Output:

Task 2:

Description:

We had to make a 2x1 MUX, where the select bit decided whether the output would be ‘a & b’ or ‘a ¦ b’. This was to be done using behavior level Verilog.

Code:

module twoonemux(a,b,select,c); input a,b; input select; output reg c; always @ * begin if (select == 0) c = a & b; else if (select == 1) c = a + b; end endmodule module testbench(); reg a,b; reg select; wire c; twoonemux uut(a,b,select,c); initial begin a = 0; b = 0; select = 0; #10; a = 0; b = 1; select = 0; #10; a = 1; b = 0; select = 0; #10; a = 1; b = 1; select = 0; #10; a = 0; b = 0; select = 1; #10; a = 0; b = 1; select = 1; #10; a = 1; b = 0; select = 1; #10; a = 1; b = 1; select = 1; #10; end endmodule

Output:

Task 3:

Description:

In this task, we had to make a two-bit magnitude comparator in Verilog, using behavior level Verilog. It was then simulated using a testbench. I took two 2-bit binary numbers and compared them. There were three variables for the equal, greater and lesser scenarios, and whichever one was fulfilled gave a true output.

Code:

module comparator(a,b,greater,equal,lesser); input [1:0] a,b; output reg greater, equal, lesser; always @ * begin greater = 0; equal = 0; lesser = 0; if (a > b) greater = 1; else if (a == b) equal = 1; else lesser = 1; end endmodule module testbench(); reg [1:0] a,b; wire greater, equal, lesser; comparator uut(a,b,greater,equal,lesser); initial begin a = 2'b01; b = 2'b10; #10; a = 2'b11; b = 2'b10; #10; a = 2'b00; b = 2'b01; #10; a = 2'b10; b = 2'b10; end endmodule

Output: