Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#7)

Arithmetic Logic Unit

Student Name: HAIDER IRFAN     Roll No: 24I-6505    Section: A

Date performed: October 5, 2026

MARKS AWARDED: ________ / 10

Date and Signature of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

In this task, we had to design a 1-bit ALU using Verilog, at all three levels of abstraction: gate level, data-flow level and behavioral-level Verilog. A testbench was included in the behavioral Verilog version to monitor how the code works. The Logisim circuit is also attached.

Code:

Behavioral Level: module alu1bit(a, b, carryin, binv, select, result, carryout); input a, b; input carryin; input binv; input [1:0] select; output reg result; output reg carryout; reg m; reg sum; reg cout; always @ * begin m = b ^ binv; {cout, sum} = a + m + carryin; result = 0; carryout = 0; if (select == 2'b00) result = a & b; else if (select == 2'b01) result = a ¦ b; else if (select == 2'b10) begin result = sum; carryout = cout; end else result = 0; end endmodule module testbenchalu(); reg a, b, carryin, binv; reg [1:0] select; wire result, carryout; alu1bit uut(a, b, carryin, binv, select, result, carryout); initial begin select = 2'b00; a = 0; b = 1; carryin = 0; binv = 0; #10; a = 1; b = 1; #10; select = 2'b01; a = 0; b = 0; #10; a = 1; b = 0; #10; select = 2'b10; binv = 0; carryin = 0; // addition a = 0; b = 1; #10; a = 1; b = 1; #10; binv = 1; carryin = 1; // subtraction a = 0; b = 1; #10; a = 1; b = 0; #10; end endmodule Dataflow Level: module alu1bit(a, b, carryin, binv, select, result, carryout); input a, b; input carryin, binv; input [1:0] select; output result; output carryout; wire m; wire x1, sum; wire c1, c2, cout; wire andans, orans; wire seland, selor, selmath; assign m = b ^ binv; assign x1 = a ^ m; assign sum = x1 ^ carryin; assign c1 = a & m; assign c2 = x1 & carryin; assign cout = c1 ¦ c2; assign andans = a & b; assign orans = a ¦ b; assign seland = ~select[1] & ~select[0]; assign selor = ~select[1] & select[0]; assign selmath = select[1] & ~select[0]; assign result = (seland & andans) ¦ (selor & orans) ¦ (selmath & sum); assign carryout = selmath & cout; endmodule Gate Level: module alu1bit(a, b, carryin, binv, select, result, carryout); input a, b; input carryin, binv; input [1:0] select; output result; output carryout; wire m; wire x1, sum; wire c1, c2, cout; wire andans, orans; wire seland, selor, selmath; xor (m, b, binv); xor (x1, a, m); xor (sum, x1, carryin); and (c1, a, m); and (c2, x1, carryin); or (cout, c1, c2); and (andans, a, b); or (orans, a, b); wire n1, n0; not (n1, select[1]); not (n0, select[0]); and (seland, n1, n0); and (selor, n1, select[0]); and (selmath, select[1], n0); wire r1, r2, r3; and (r1, seland, andans); and (r2, selor, orans); and (r3, selmath, sum); or (result, r1, r2, r3); and(carryout, selmath, cout); endmodule

Output:

Task 2:

Description:

In this task, I made a 4-bit ALU by cascading four 1-bit ALUs in Verilog. An SLT function was also added to the original 1-bit ALU. A three bit input was used since there were five outputs, and four out of the eight possible combinations were mapped onto the 1-bit ALUs. A testbench was also written to test out the functioning of the code.

Code:

module alu1bit(a, b, carryin, binv, less, select, result, carryout); input a, b; input carryin; input binv; input less; input [1:0] select; output reg result; output reg carryout; reg m; reg sum; reg cout; always @* begin m = b ^ binv; {cout, sum} = a + m + carryin; result = 0; carryout = 0; if (select == 2'b00) result = a & b; else if (select == 2'b01) result = a ¦ b; else if (select == 2'b10) begin result = sum; carryout = cout; end else result = less; end endmodule module alu4bit(a, b, op, result, CF, SF, ZF, OF); input [3:0] a, b; input [2:0] op; output reg [3:0] result; output reg CF, SF, ZF, OF; wire binv; wire [4:0] carry; wire [3:0] tempresult; wire lessbit; wire overflow; reg [1:0] select; always @* begin if (op == 3'b000) select = 2'b00; else if (op == 3'b001) select = 2'b01; else if (op == 3'b111) select = 2'b11; else select = 2'b10; end assign binv = (op == 3'b110 ¦¦ op == 3'b111); assign carry[0] = binv; alu1bit bit0(a[0], b[0], carry[0], binv, lessbit, select, tempresult[0], carry[1]); alu1bit bit1(a[1], b[1], carry[1], binv, 1'b0, select, tempresult[1], carry[2]); alu1bit bit2(a[2], b[2], carry[2], binv, 1'b0, select, tempresult[2], carry[3]); alu1bit bit3(a[3], b[3], carry[3], binv, 1'b0, select, tempresult[3], carry[4]); assign overflow = carry[3] ^ carry[4]; assign lessbit = tempresult[3] ^ overflow; always @ * begin result = tempresult; CF = 0; SF = result[3]; ZF = (result == 4'b0000); OF = 0; if (op == 3'b010 ¦¦ op == 3'b110 ¦¦ op == 3'b111) begin CF = carry[4]; OF = overflow; end end endmodule module testbench(); reg [3:0] a, b; reg [2:0] op; wire [3:0] result; wire CF, SF, ZF, OF; alu4bit uut(a, b, op, result, CF, SF, ZF, OF); initial begin a = 5; b = 3; op = 3'b000; // and #10; a = 5; b = 3; op = 3'b001; // or #10; a = 5; b = 3; op = 3'b010; // add #10; a = 5; b = 3; op = 3'b110; // or #10; a = 3; b = 5; op = 3'b111; // slt #10; end endmodule

Output:

Task 3:

Description:

In this task, we had to use parametrization to make the ALU code generalized for any N number of bits. The Verilog code performs addition, subtraction and the basic logical operations.

Code:

module alu#(parameter N = 4) ( input [N-1:0] a, input [N-1:0] b, input [2:0] control, output reg [N-1:0] result ); always @* begin case (control) 3'b000: result = a + b; // add 3'b001: result = a - b; // sub 3'b010: result = a & b; // and 3'b011: result = a ¦ b; // or 3'b100: result = ~a; // not 3'b101: result = a ^ b; // xor default: result = 0; endcase end endmodule module testbench(); reg [3:0] a, b; reg [2:0] control; wire [3:0] result; alu #(4) uut(a, b, control, result); initial begin a = 5; b = 3; control = 3'b000; #10; control = 3'b001; #10; control = 3'b010; #10; control = 3'b011; #10; control = 3'b100; #10; control = 3'b101; #10; end endmodule

Output:

Task 4:

Description:

Had to make a fully functional 32-bit ALU using Verilog. The ALU performs ten operations: addition, subtraction, AND, OR, XOR, signed and unsigned comparison, logical left shift, logical right shift and arithmetic right shift. The operation is selected using a 4-bit control input.

Code:

module alu (a, b, alu_op, result); input [31:0] a, b; input [3:0] alu_op; output reg [31:0] result; output zero; always @* begin case (alu_op) 4’b0000: result = a + b; // add 4’b0001: result = a – b; // sub 4’b0010: result = a & b; // and 4’b0011: result = a ¦ b; // or 4’b0100: result = a ^ b; // xor // slt: signed less than 4’b0101: begin if (a[31] != b[31]) result = a[31] ? 1 : 0; else if ($signed(a) < $signed(b)) result = 1; else result = 0; end // sltu: unsigned less than 4’b0110: begin if (a < b) result = 1; else result = 0; end 4’b0111: result = a << b[4:0]; // sll 4’b1000: result = a >> b[4:0]; // srl 4’b1001: result = $signed(a) >>> b[4:0]; // sra default: result = 0; endcase end assign zero = (result == 0); endmodule

Task 5:

Description:

We had to make a 32-bit ALU circuit in Logisim, which has been implemented below. Bit extenders and splitters were used to solve issues of incompatible widths.

Output: